| Project Name | Ritlecitinib Tosylate | Project Code: RIT | Proposed Date: |
| --- | --- | --- | --- |
| Product Name | 3‐Chloropropanoyl chloride | Product Code: RIT-S3 | PS No.: |

PRODUCT INFORMATION

| CATEGORY | Key Starting Material |
| --- | --- |
| STRUCTURE |  |
| CHEMICAL/ IUPAC NAME | 3‐Chloropropanoyl chloride |
| MOLECULAR FORMULA | C3H4Cl2O |
| MOLECULAR WEIGHT | 126.97 g/mol |
| CAS NUMBER | 625-36-5 |
| PACKING CONDITION | Pack the material in HDPE lined MS container and seal it properly. |
| STORAGE CONDITIONS | Store in dry and well-ventilated area at temperature not exceeding 25 oC and away from the sources of heat, light, spark and moisture. |
| HANDLING PRECAUTIONS | Handling should be done in a ventilated area and under nitrogen atmosphere. During handling of material, the person should wear all the PPEs. |

| Project Name | Ritlecitinib tosylate | Project Code: RIT | Proposed Date: |
| --- | --- | --- | --- |
| Product Name | 3‐Chloropropanoyl chloride | Product Code: RIT-S3 | PS No.: |

| S. No | Test Parameter | Acceptance Criteria |
| --- | --- | --- |
| 1. | Description | Clear colorless to pale yellow color liquid |
| 2. | Identification |  |
|  | a. By IR | The IR spectrum of the sample should be concordant with the IR spectrum of RIT-S3 reference /working standard |
|  | b. By GC | The retention time of major peak in the chromatogram of the sample solution should correspond to the major peak in the chromatogram of the standard solution-1 in the test for Assay by GC |
| 3. | Related substances by GC (%) |  |
|  | a. RIT-S3 Impurity-A (~1.21 RRT) * | Not more than 1.0 |
|  | b. RIT-S3 Impurity-C | Not more than 0.50 |
|  | c. RIT-S3 Impurity-D | Not more than 0.10 |
|  | d. RIT-S3 Impurity-E | Not more than 0.10 |
|  | e. RIT-S3 Impurity-F | Not more than 0.10 |
|  | f. RIT-S3 Impurity-H | Not more than 0.10 |
|  | g. 3-Chloropropionic anhydride (~1.96 RRT) * | Not more than 1.50 |
|  | Any unspecified impurity | Not more than 0.2 |
|  | Total impurities | Not more than 3.0 |
| 4. | Assay by GC (% w/w, on as such basis) | Not less than 97.0 and not more than 102.0 |

*Note: RIT-S3 is moisture sensitive, which readily absorbs moisture from the air and interconversion with each other to the formation of 3-Chloropropionic acid (1.21 RRT impurity) and 3-Chloropropionic anhydride (1.96 RRT impurity), So quantification of these impurities will not be appropriate. These impurities will be used in validation for specificity purpose only.

Reporting of these impurities 3-Chloropropionic acid (1.21 RRT impurity) and 3-Chloropropionic anhydride (1.96 impurity) is by % area normalization without RRF correction.

| Impurity Code | Structure of the impurity | Chemical/IUPAC name of the impurity |
| --- | --- | --- |
| RIT-S3 Impurity-A (~1.21 RRT) |  | 3-Chloropropanoic acid |
| RIT-S3 Impurity-C |  | Prop‐2‐enoyl chloride (Acrolyl chloride) |
| RIT-S3 Impurity-D |  | Benzoyl chloride |
| RIT-S3 Impurity-E |  | (Chloromethyl)benzene |
| RIT-S3 Impurity-F |  | (Trichloromethyl)benzene |
| RIT-S3 Impurity-H |  | 2‐Chloropropanoyl chloride |
| ~RRT 1.96 |  | 3-Chloropropionic anhydride |